- Quick Start

  - Equipment Configuration

Fig 1 Front panel of KV-8000

Device connection

[Equipment list]

・PC

・CPU unit: KV-8000

・Communication unit: KV-XL202

・Power unit: CA-U4

・Source meter: 2400s

・RS-232c cable (Need to cutting)

・USB cable

・Ethernet cable

・3-terminals power cable

Set the mode switch on the KV-8000 to PROG (Fig 1).

Connect the above devices as shown in Fig 2.

After cutting the RS-232c cable, attach the cut section to the KV-XL202 Port1. The RS-232c cable currently in use can be connected with the color pattern shown in the lower right of Fig 2. It is recommended that a continuity check be performed when changing cables.

Fig 2 Device connection

Fig 3 Photo of actual configuration

    - 2400s Settings

[Communication settings]

- Press the Menu button.

- Select Communication and press Enter

- Select RS-232 and press Enter

- Select BAND and press Enter. Select 9600 and press Enter.

- Select BITS and press Enter. Select 8 and press Enter.

- Select PARITY and press Enter. Select EVEN and press Enter.

- Select THERMINATOR and press Enter. Select <CR+LF> and press Enter.

- Select FLOW-CTRL and press Enter. Select XON-XOFF and press Enter.

- Press the Exit button twice to return.

Fig 4 Front panel of 2400s

  - Create KV-studio Project

- Launch KV-studio.

- Click on [File] -> [New project].

- Enter Project name, PLC model, Project location. (Fig 5)

- Click OK to create the project.

- When the “Automatically set the operation recorder setting?” dialog box appears, select Yes. (*This setting is optional.)

- When the “Setup unit setting info now?” dialog box appears, select No.

Fig 5 Create New Project window

  - Configure PLC Settings

- Click on [Tool] -> [Clear PLC].

- Clear PLC: Select a Clear All and Execute.

- Project tree: Right click on [Unit configuration] -> Click on [Unit Editor].

- Unit Editor: Click on [File] -> [Read Unit config. from PLC].

- Unit Editor: When the “Unit settings are read in the initialization status.” Dialog box appears, select Yes.

- Unit Editor: Select KV-8000 and Set [FTP server] to “Used”. (Fig 6)

- Unit Editor: Select KV-XL202 and Change to the following settings. (Fig 7)

・Port1, Operation mode: Select “Non-procedure mode”.

・Port1, Device allocation: “DM”. (default)

・Port1, Interface: “RS-232C”. (default)

・Port1, Baudrate: “9600bps”. (Match with 2400s settings)

・Port1, Data bit length: “8 bits”. (Match with 2400s settings)

・Port1, Start bit: “1 bits”. (default)

・Port1, Stop bit: “1 bits”. (default)

・Port1, Parity: “Even”. (Match with 2400s settings)

・Port1, Checksum: “none”. (default)

・Port1, RS/CS flow control: “ON”. (Match with 2400s settings)

- Unit Editor: Click on [Apply]. And click on [OK].

Fig 6 Configuration of KV-8000

Fig 7 Configuration of KV-XL202

Fig 8 Device Comment

Register unit device

- Click on [View] -> [Device comment edit window].

- Device comment edit: Click on [Detail].

- Device comment edit: Click on [Register unit device].

- Register unit device: Select KV-8000 and KV-XL202.

- Register unit device: Click on [Reg].

- Device comment edit: Verify that the unit device is registered. And exit. (Fig 8)

  - Create a Ladder Program

Import a basic program already created. The sample projects referenced exist in the following directories.

[Sample project]

C:\....\PLC_CommunicationTo2400S\Documents\Sample1\Sample1.kpr

- Writing: recorder setting: continue sending(No: this setting is optional.)

- A

  - Load settings from PLC

  - Writing settings and programs

  - Displaying device values

  - RS-232c communication of KV-XL202

- Aaa

- Aaa

- Aaa